Introduction

Spices, Inc. maintains documented cleaning procedures to ensure equipment and utensils are cleaned and sanitized in a manner appropriate for a dry spice manufacturing environment. Cleaning is conducted to prevent product contamination and allergen cross-contact.

All cleaning activities are recorded on designated logs and verified by Quality Assurance.

Scope

This procedure applies to:

- Rotary Blender (allergen and non-allergen use)

- Two spice grinders (non-allergen only)

- Dish room operations

- High-heat dishwasher

- Three-compartment sink

- All utensils used in production, blending, packaging, and handling

Responsibilities

Production Staff

- Perform cleaning as required

- Complete cleaning logs

Operations Manager

- Ensures cleaning is completed after required events

- Ensures employees are trained

Quality Assurance (QA)

- Reviews cleaning logs

- Conducts visual verification

- Maintains sanitation records

Cleaning Frequency Requirements

| Equipment / Area | Frequency | Record |
| --- | --- | --- |
| Rotary Blender | After every batch | Rotary Blender Cleaning Log |
| Grinder #1 & #2 | After every batch | Grinder Cleaning Log |
| Utensils | After each use | Dishwasher / Sink Log |
| Dishroom | Daily | Sanitation Log |

## Equipment Cleaning Methods

### Rotary Blender

Used for both allergen and non-allergen products.
A full wet cleaning is performed after every batch.

Cleaning includes:

- Removal of remaining product

- Hot water rinse

- Detergent wash with rotation to clean all internal surfaces

  - Simple Green 1 to 10 ratio

- Final rinse

- Air drying

- Exterior wipe-down using Simple Green

  - Air dry – no additional rinse

Cleaning is documented on the Rotary Blender Cleaning Log

Grinders

Designated for non-allergen products only.
Cleaned after every batch.

Cleaning includes:

- Removal of remaining spice material

- Dry cleaning using brushes and vacuum

- Wipe-down of contact surfaces as needed

  - Purell Foodservice Surface Sanitizer

- Visual inspection prior to next use

Cleaning is documented on the Grinder Cleaning Log.

Utensil Control – One Touch System

Spices, Inc. utilizes a One Touch System:

- Clean utensils are stored in designated clean areas.

- Once used, utensils are immediately placed in dirty bowls.

- Utensils are not returned to clean storage without full washing and sanitizing.

This system prevents cross-contamination and allergen cross-contact.

## Dishroom Procedures

### High-Heat Dishwasher

Used for general production utensils (non-allergen-specific items).

- Pre-scrape

- Wash using high-heat cycle (minimum 160°F)

- Air dry

- Store in designated clean area

### Three-Compartment Sink

Used for:

- Allergen-contact utensils

- Large equipment that does not fit in dishwasher

Setup:

- Rinse water

- Dawn dish soap wash

  - 1 cup Dawn Dish soap to 15 Gallons of water

- Bleach sanitizing solution – 50 ppm

  - Documented on Dish room records

Utensils are:

- Washed

- Sanitized

- Air dried

- Returned to designated storage

Bleach concentration is monitored to ensure effectiveness.

Chemicals Used

- Dawn dish soap (detergent)

- Bleach concentrate (sanitizer for 3-compartment sink)

- Approved surface sanitizer for equipment wipe-down

All chemicals are:

- Stored properly

- Labeled

- Used according to manufacturer instructions

## Verification & Records

Cleaning is verified by:

- Visual inspection

- Review of cleaning logs by QA

Required records:

- Rotary Blender Cleaning Log

- Grinder Cleaning Log

- Dishwasher / Sink Log

- Sanitation Log

Corrective Actions

If cleaning is found to be ineffective:

- Equipment is re-cleaned immediately.

- Production does not resume until verified clean.

- Root cause is determined if issue is recurring.

- Retraining is conducted if necessary.

- QA documents corrective action.

| Revision Date = Version # | Changes |
| --- | --- |
| 2025-03-24 | New program. |
| 03/16/2026 | Reviewed and revised |