INTRODUCTION

The purpose of conducting an environmental monitoring program is to identify any areas within the plant processing environment which may harbor pathogens and serve as a source of product contamination.  Clearly identifying these areas allows clean up and sanitation efforts to be refocused so that the potential sources of contamination are eliminated.

SCOPE

Spices Inc. is a dry processing facility with no use of water as an ingredient or processing aid, and very limited use of water, under controlled application an containment, for the purpose of cleaning floors, utensils, and allergen removal from shared contact surfaces. As such, this program is limited to routine environmental monitoring using the hygiene indicator organism Enterobacteriaceae (EB) to verify the effectiveness of cleaning and sanitation practices for controlling Salmonella species.

RESPONSIBILITY

SQF Practitioner, (COO) for written program; SQF practitioner, (COO) or designee for sampling, interpretation of results, and corrective actions.

PROCEDURE

# Background

# Spices Inc. repacks bulk spices into smaller containers for use by consumers, foodservice, and further manufacturers. Grinding and blending of spices is also conducted. All raw materials received are ready-to-eat (RTE). No antimicrobial intervention, process or ingredient is applied to the spices. All products are of low water activity, well below Aw 0.85, and will not support pathogen growth. As such, the process and product is considered “moderate risk,” with contamination by Salmonella species being of primary concern due to the dry nature of the processing environment and product.

# Food contact surfaces are cleaned daily using a combination of dry removal of product residues followed by wet cleaning and sanitizing. Wet cleaning includes the use of approved cleaning agents (e.g., Simple Green) followed by alcohol-based sanitizing (Alpet 2). Washable, reusable cleaning cloths are used and are laundered according to established sanitation procedures. Following sanitizing, food contact surfaces are left to air dry prior to use or reassembly to prevent recontamination and to ensure the sanitizer has adequate contact time.

# Food contact surfaces are visually inspected during pre-operational inspections. Wet cleaning and sanitizing is performed on all food contact surfaces regardless of allergen status.

# Food contact surfaces and equipment include, but are not limited to: production tables, scales, blender (when used), and utensils. Utensils are cleaned using a high-heat commercial dishwasher. Allergen-designated utensils are first washed with Dawn dish soap and then sanitized in the high-heat dishwasher.

# Environmental Monitoring Approach

# Environmental monitoring is conducted using Enterobacteriaceae (EB) as a sanitation and hygiene indicator organism for Salmonella species.

# Sampling locations and frequency are determined based on the Environmental Monitoring Risk Assessment. The risk assessment evaluates areas of the facility based on factors such as proximity to food contact surfaces, employee and material traffic, equipment design, and potential for contamination.

# Each area is assigned a relative risk level, which is used to guide both the selection of sampling sites and the frequency of testing. Higher risk areas are sampled more frequently, while lower risk areas are sampled less frequently or on a rotating basis.

# Sampling locations are not fixed and may be adjusted over time based on updates to the risk assessment, environmental monitoring results, and observed trends. The Environmental Monitoring Risk Assessment serves as the primary document for determining where and how often environmental monitoring is conducted.

# Sampling may include Zone 1 (food contact), Zone 2, Zone 3, and Zone 4 surfaces, as defined below:

# Zone 2: Surfaces in close proximity to food contact surfaces (e.g., table edges, equipment framework)

# Zone 3: Non-food contact surfaces within processing areas (e.g., floors, walls, overheads)

# Zone 4: Areas outside of processing zones (e.g., hallways, warehouse, employee areas)

Designated Sampler

Sampling will be completed by the Chief Operations Officer or designee.

Sample Quantity and Frequency

The number of sites tested and testing frequency are determined by the Environmental Monitoring Risk Assessment. Testing frequency may be adjusted based on environmental monitoring results or identified risks.

Sampling Method and Record Keeping

Environmental samples are collected using Hygiena MicroSnap™ EB rapid detection devices according to the manufacturer’s instructions.

Clean, disposable gloves are worn during sampling. Each device is labeled with the sampling location. Samples are incubated on-site and results are recorded in the Environmental Monitoring Log.

Corrective actions taken in response to EB results exceeding the acceptable limit (<10 CFU) are documented in the corrective action log.

Sample Time Selection

Environmental monitoring samples are collected at the beginning of the first production shift prior to the start of operations. The facility operates a single production shift.

Sample Testing

Environmental samples are tested on-site using Hygiena MicroSnap™ EB rapid test kits according to manufacturer instructions. Results are reviewed and recorded by the SQF Practitioner or designee.

Sample Data Collection and Trending

Test results are collected and reviewed at least annually to identify trends. The frequency of trend analysis may be adjusted based on program performance and results.

Corrective Action Program/ Enhanced Sampling

If an Enterobacteriaceae (EB) result exceeds the established acceptable limit (<10 CFU), additional samples are taken immediately, prior to cleaning, to assist in identifying the source.

The affected area is re-cleaned and sanitized and re-tested on the next production day to verify effectiveness.

If repeat unsatisfactory results occur, cleaning procedures will be reviewed and modified as needed, and additional corrective actions will be documented.

DOCUMENT HISTORY

NOTE: Only record revisions to this document. Reviews are recorded only in the central Document Register.

| Revision Date = Version | Changes |
| --- | --- |
| 2025-02-04 | New program. |
| 03/16/2026 | Reviewed and Revised |