## INTRODUCTION

This document defines the methods used for identifying and tracking raw materials, ingredients, packaging materials, work in progress, process inputs, rework, and finished products during all stages of production and storage.

## SCOPE

This document applies to all raw materials, in-process materials, and finished products that are identified, lot-coded, tracked, and subject to recall at Spices Inc. Traceability is maintained using in-house lot numbers and date-based records.

RESPONSIBILITY

Chief Operations Officer (COO), Quality Assurance Manager, Inventory Control Manager

PROCEDURE

## Raw Material Receipt & Identification

### Receiving

### Raw materials are received via freight carriers or FedEx Ground.

### Upon receipt, each raw material is inspected and logged in the Lot Number Book.

### Receipt dates for ingredients and food-contact packaging/materials are recorded on receiving records

### In-House Lot Code Assignment

- Each raw material is assigned an in-house lot number at receipt.

Lot code format:

[SKU] – [Supplier Code] – [2-Digit Day] – [Month Letter] – [2-Digit Year]

Month lettering system:

- A = January

- B = February

- C = March

- (continues sequentially through the year)

Example:

100102BY21A26

### Lot Number Recording

- The lot number is recorded in the Lot Number Book when received and approved.

- When the first container of that batch is opened, the lot is marked as with a start date.

- All product sold after the open date is traceable and assigned to that lot until a new lot is started.

## Raw Material Use in Production

### Use in Blends

- When a raw material is used in a blend, the specific in-house lot number is recorded on the recipe sheet.

- This provides one-step forward and one-step back traceability.

## In-House Grinding

### Grinding Process

- Some raw materials are ground in-house.

- Grinding starts using the original raw material lot number.

### Ground Product Lot Number

- Once grinding begins, the ground product is assigned a new in-house lot number.

- The supplier code for ground products is SG (Spices Inc. – Ground).

- A start date is recorded for each ground lot.

### Lot Assumption

- All orders filled from the start date until the next lot start date are presumed to originate from that ground lot.

## Spice Blends (Finished Products)

### Blend Lot Numbers

- Each spice blend is assigned its own in-house lot number.

- The supplier code used for blends is SB (Spices Inc. – Blend).

- All raw material and ground ingredient lot numbers used are recorded on the recipe sheet.

### Start Date Tracking

- A start date is recorded for each blend lot.

- All orders shipped from that start date until the next blend lot is started are presumed to originate from that lot.

## Finished Product Records & Order Tracking

- Spices Inc. maintains records of every customer order and every product shipped.

- Orders are linked to production dates and lot start dates.

- Shipping records, invoices, and production logs are retained to support traceability.

## Recall & Traceability Method

- Spices Inc. operates a date-based recall system supported by in-house lot numbers.

- In the event of a:

  - Raw material recall → affected raw lots are traced forward to:

    - Ground products

    - Spice blends

    - Finished customer orders

  - Finished product recall → affected orders are identified based on:

    - Lot start date through the next lot start date

This system allows Spices Inc. to identify:

- Affected raw materials

- All finished products containing those materials

- All customers who received affected product

within the timeframes required by SQF.

## Spices, Inc. does not use rework.

## The traceability system is reviewed at least annually as part of the product withdrawal/recall system review and mock recall testing.

## Records Maintained

- Lot Number Book

- Recipe Sheets

- Grinding Logs

- Picklots

- Ship Station

- Order Master

| Revision Date = Version # | Changes |
| --- | --- |
| 2026-01-20 | New program. |
| 02/11/2026 | Reviewed and revised |