PURPOSE

To minimize the risk of allergen cross-contact and undeclared allergens through the adoption of best practices for allergen receipt, storage, cleaning, and labeling.

SCOPE

All manufacturing and storage spaces, ingredients, and finished products.

RESPONSIBILITY

SQF Practitioner for written program and training; Production and Warehouse supervisors for implementation.

PROCEDURE

Ingredient Evaluation

A Product Supplier Master spreadsheet is maintained and updated regularly to document all raw materials and approved suppliers. Within this spreadsheet, ingredients containing known allergens are clearly documented and highlighted. A review of current ingredients shows the following allergenic ingredients present at this facility:

- Soy, Treenuts (Almond, Hazlenut), Sesame, Dairy, Peanuts

This allergen list is reviewed during raw material approval and whenever new ingredients are introduced.

Other Raw Material Evaluation

Spices Inc. utilizes a food-grade grease on the external, non-product contact surfaces of the rotary blender. This processing aid does not contain any of the nine major food allergens. The grease is used only on non-food contact surfaces and is controlled through preventive maintenance and sanitation procedures.

Proposed introduction of new processing aids will include an evaluation for allergens. The introduction of new allergens from any source will be avoided where possible. If new allergens are introduced, this allergen management plan will be reassessed accordingly.

Various allergens may be brought into the facility through employee lunches, snacks, or company-sponsored meals. To control this risk, all food consumption is restricted to designated break areas away from production and food contact surfaces. Employees are required to remove aprons or protective garments before entering break areas, and upon returning to production areas they must wash and sanitize their hands in accordance with the facility’s hygiene procedures before resuming work. All employees receive training in allergen awareness and proper hygiene practices to prevent cross-contact between personal food items and production activities.

Allergen Identification and Awareness

Training

All employees will be trained in allergen awareness and pertinent elements of this program upon hire. This program will also be reviewed at least annually with all employees.

Receiving and Identification System

The Receiver is provided with a list of target allergens and has access to the list of allergen-containing materials in the Raw Material Register. Upon receipt, all inbound ingredient pallets are marked with an allergen placard indicating the types of allergens present. If multiple allergens are present, multiple placards are applied. Allergen identification remains with the pallet throughout storage and use according to the following table:

| Allergen | Color Code |
| --- | --- |
| Soy | Blue |
| Dairy | Purple |
| Sesame | Green |
| Treenut | Yellow With specific species on label |
| Peanuts | Red |

The allergen placard remains with the pallet throughout storage.

The allergen color-coding scheme is posted throughout the facility in key areas including: Receiving, Warehousing, Packaging, Grinding/Blending.

Ingredient Storage

Allergenic ingredients will be stored in segregated areas identified according to the allergens in that area. Overflow volumes of allergenic ingredients may be stored in alternative locations if pallets are stored on the lowest level, and allergen identification placards are readily visible.

Materials containing allergens are stored to prevent cross-contact with non-allergen materials. Controls include physical separation, intact outer packaging, sealed inner plastic liners secured with zip ties, and visible allergen identification. Materials containing allergens are not stored above non-allergen materials.

Production Scheduling and Sanitation

Spices Inc. operates as a dry-processing facility using shared equipment. Dedicated allergen equipment is not utilized. Production sequencing and sanitation controls are the primary methods used to minimize allergen cross-contact.

Cleaning methods for direct contact (zone 1) and adjacent (zone 2) surfaces are determined based on allergen risk and equipment design and may include dry cleaning, wet cleaning, and sanitizing as appropriate. At a minimum, all zone 1 and zone 2 surfaces must be visually examined after cleaning for the absence of visible residue.

Allergen changeover cleaning effectiveness is verified through visual inspection and allergen-specific swabbing. Spices Inc. maintains allergen test kits for each allergen handled at the facility. Results are reviewed and maintained as part of sanitation records.

Sanitation activities include dry wiping followed by wet cleaning using Simple Green Pro D where required, and final sanitizing using Alpet D2 alcohol-based sanitizer, in accordance with established allergen cleaning procedures. Controls to prevent allergen cross-contact are implemented during cleaning and in the utensil wash room, including the segregation of allergen-designated utensils and scoops, separate washing of allergen utensils prior to sanitizing in the high-heat dishwasher, use of designated allergen aprons and gloves, and controlled cleaning of equipment and food contact surfaces.

Allergen cleaning and verification is recorded on appropriate SSOP logs. Allergen swabbing is performed as needed to verify cleanliness and sanitation. Allergen swabbing is recorded on Allergen Testing Log.

Label Reconciliation

Finished product labels are generated by order and are not created on a batch-by-batch basis. Label accuracy is verified prior to production through review of the pick lot associated with each order. Verification includes confirmation that the correct label is selected for the product and that all required information, including allergen declarations, is accurate. Label verification is documented by initialing the pick lot. Labels are printed and used per order and are not retained as part of production records.

Rework

Rework is addressed by the Product Rework program. Currently, no product is reworked.

Nine Food Allergens Identified by the FDA

- Milk

- Eggs

- Fish (e.g. bass, flounder, cod.) – specific to each species

- Crustacean shellfish (e.g. crab, lobster, shrimp) – specific to each species

- Tree nuts (e.g. almonds, walnuts, pecans, coconut) – specific to each species

- Peanuts

- Wheat

- Soy

- Sesame

Additional allergens are identified by other countries. However, Spices Inc. does not export any product currently.

May Contain or Similar Labeling; Gluten Labeling

US law does not require that finished products labels indicate whether allergenic materials are handled on shared equipment or within the plant. Such labeling may be present, if required by private-label clients, but is not consistently applied to all product labels otherwise. In addition, US law does not require specific declaration of gluten or gluten-containing materials on labels, apart from gluten from wheat, in which case “wheat” is the required label declaration. Therefore, “gluten” labeling will only be applied if it is a customer requirement.

Label verification includes confirmation that all allergens present in the product formulation are accurately declared on the finished product label.

Allergen Production, Blending, and Filling Controls

Filling Cell Controls

Spices Inc. utilizes a kanban-based production system with multiple designated filling cells on the production floor. Seven filling cells are in operation. Filling Cell 1 is designated exclusively for the handling and filling of allergen-containing products and is not used for non-allergen filling activities.

Within Cell 1, allergen ingredients and finished products are kept separated to prevent cross-contact. Similar allergens are filled together where feasible to minimize changeovers. When transitioning between different allergens, employees change disposable gloves and wipe down the work surface with a disposable paper towel and Simple Green prior to continuing filling activities. The work surface is cleaned and sanitized at the end of each production day.

Employees assigned to allergen handling are trained in allergen control procedures. Disposable gloves are changed immediately after completing work at the allergen table, and a designated blue apron is required while handling allergens. Blue aprons used for allergen handling are not permitted to be worn at any other filling cells.

Blending Room Allergen Controls – Hand Blending

All products are blended in the blend room using shared equipment and workspaces. When allergens are blended by hand using bowls, allergen-containing ingredients are handled at a designated area with adequate distance from non-allergen blending activities to prevent cross-contact.

Blending Room Allergen Controls – Rotary Blender

The rotary blender is used to produce both allergen and non-allergen blends. When allergen-containing blends are produced, the allergen ingredient is added as the final ingredient to minimize cross-contact risk. Employees are required to wear the designated blue apron while handling allergen ingredients and for the remainder of the blend.

Designated Allergen Utensils and Tool Controls

Designated scoops and funnels are used exclusively when handling allergen-containing ingredients during filling and blending activities, including hand blending and rotary blending. Allergen-designated scoops and funnels are clearly identified and are not shared with non-allergen products.

Upon completion of allergen handling activities, allergen-designated scoops and funnels are washed separately from non-allergen utensils in accordance with sanitation procedures to prevent cross-contact.

DOCUMENT HISTORY

NOTE: Only record revisions to this document. Reviews are recorded only in the central Document Register.

REFERENCES
SQF Fundamentals for Manufacturing - Intermediate EDITION 1.1/ 2.8 Allergen Management

| Revision Date = Version # | Changes |
| --- | --- |
| 2026-01-29 | New program. |
| 02/07/2026 | Reviewed |
| 03/16/2026 | Revised |